Criteria for all bay segments are expressed as annual geometric mean (AGM) values not to be exceeded more than once in a three year period. Nutrient and nutrient response values do not apply to tidally influenced areas that fluctuate between predominantly marine and predominantly fresh waters during typical climatic and hydrologic conditions.

Table 1. NNC values for St. Andrew and St. Joseph Bays

| Water Body | Total Phosphorus | Total Nitrogen | Chlorophyll a | Source of Criteria |
| --- | --- | --- | --- | --- |
| East Bay | 0.016 mg/L as AGM | 0.33 mg/L as AGM | 3.9 µg/L as AGM | FAC 62-302.532 |
| North Bay | 0.014 mg/L as AGM | 0.28 mg/L as AGM | 3.1 µg/L as AGM | FAC 62-302.532 |
| St. Andrew Bay | 0.019 mg/L as AGM | 0.34 mg/L as AGM | 3.7 µg/L as AGM | FAC 62-302.532 |
| West Bay | 0.017 mg/L as AGM | 0.35 mg/L as AGM | 3.8 µg/L as AGM | FAC 62-302.532 |
| Deer Point Lake | 30 µg/L as AGM | 930 µg/L as AGM | 6 µg/L as AGM | FAC 62-302.530 |
| Lake Powell | 19 µg/L as AGM | 611 µg/L as AGM | NA | Bachmann et al., 2012 |
| Lullwater | 90 µg/L as AGM | 1910 µg/L as AGM | 20 µg/L as AGM | FAC 62-302.530 |
| Lake Martin | 90 µg/L as AGM | 1910 µg/L as AGM | 20 µg/L as AGM | FAC 62-302.530 |
| Crooked Island Sound | 0.019 mg/L as AGM | 0.34 mg/L as AGM | 3.7 µg/L as AGM | FAC 62-302.532 |
| Lakes |  |  |  |  |

Bachmann, R. W., Bigham D. L., Hoyer M. V., Canfield D. E, Jr. 2012c. A strategy for establishing numeric nutrient criteria for Florida lakes. Lake Reserv Manage. 28:84-92.

## Linking the bayous with major bay segments

- In ArcGIS, if the current sampling boundary layer does not have a major segments column (West Bay, North Bay, East Bay, St. Andrew Bay, St. Joseph Bay), then you will need to create a new field so the bayous can be linked to each major segment.

  - New Field

    - Field Name: Major_Seg

    - Alias: Major_Seg

    - Data Type: Text

  - Assigning major segments using select by location (North Bay example)

    - Selecting the first major segment

      - Open Select by Attributes

      - Set input rows to your bay/bayou layer

      - Set selection type to New Selection

      - In the SQL expression editor: SEGMENT_NA = 'North Bay'

        - Alternatively, you can use the drop down menus

          - Where: SEGMENT_NA is equal to North Bay

      - Apply and Okay

    - Select all bayous connected to North Bay

      - Open Select By Location

      - Set input features to your bay/bayou layer

      - Set selecting features to the same layer

      - Toggle “use the selected records” ON (so it only used North Bay)

      - Set relationship = intersect

      - Apply and Okay

    - Removing any unwanted areas from the selection

      - Open Select By Attributes again

      - Change selection type = remove from current selection

      - In the drop down menus choose any SEGMENT_NA that is connected to your chose bay segment that you do not want to be connected.

      - Apply and Okay

    - Assign the major segment value

      - In the bays/bayous attribute table, open calculate field for the new Major_Seg column

      - Ensure “use the selected records” is toggled ON

      - Choose the field Major_Seg ( or your major water body field)

      - In the expression box enter “North Bay”

      - Click apply and okay. All the relevant SEGMENT_NA should now have an adjacent North Bay in the Major_Seg column

    - Repeat this process for each major segment. Ensure that any areas that were de-selected from the major segment are include in the correct segment.

## Assessing the Data

- Add a Major_Seg column to your water quality data layer. Join the water quality later and the previous bay/bayous layer via the SEGMENT_NA field.

- Export the joined layer a new layer and remove any unnecessary columns from the attribute table.