# GIS Methods: Site Projection and Buffer-Based Proximity Assessment

This document provides a step-by-step workflow that can be followed in ArcGIS Pro to (1) project sampling site layers to a distance-appropriate coordinate system, (2) identify sites lacking standard identifiers, and (3) evaluate spatial proximity between unidentified sites and sites with established identifiers using buffer analysis.

## 1. Project Spatial Data to a Projected Coordinate System

Distance-based analyses (e.g., buffers) require data to be in a projected coordinate system with linear units. All relevant point layers were projected prior to analysis.

### Coordinate System Used

NAD 1983 (2011) StatePlane Florida North FIPS 0903 (Meters)

### Steps

- Open ArcGIS Pro and load all relevant sampling site layers.

- Open the Geoprocessing pane and search for Project.

- Set the parameters as follows:

  - Input Dataset: Original sampling site layer (e.g., sites in WGS 1984)

  - Output Feature Class: New projected layer (e.g., Sites_With_ID_Project)

  - Output Coordinate System:

    - Projected → State Plane → NAD 1983 (2011) → Florida North → Meters

- Run the tool.

Note: The Project tool was used (not Define Projection) to correctly transform coordinates from a geographic to a projected system.

Repeat this step for all point layers that will be used in proximity or buffer analyses.

## 2. Identify Sites Lacking Standard Identifiers

### Fields Evaluated

Records were selected where all of the following fields contained the value NA: - Organization ID - Organization Name - Monitoring Location ID - Monitoring Location Name

### Steps

- Open the attribute table of the projected sampling sites layer.

- Select Select By Attributes.

- Build the following expression:

Organization_ID = 'NA'
AND Organization_Name = 'NA'
AND Monitoring_Location_ID = 'NA'
AND Monitoring_Location_Name = 'NA'

- Apply the selection and verify the selected records.

- Right-click the layer → Data → Export Features.

- Save the exported layer as Sites_No_ID.

### Site Identifier Assignment

For the Sites_No_ID layer, the U of F Number field was used as a provisional site identifier, as it consistently corresponded to unique sampling locations within this subset.

## 3. Create Buffer Layers Around Unidentified Sites

Buffers were used to assess how close unidentified sites were to sites with established identifiers.

### Buffer Distances

- 20 meters: Used to identify spatially coincident sites (e.g., GPS rounding or duplicate records)

- 50 meters: Used to identify spatially proximate sites that may represent distinct but closely located sampling locations

### Steps (repeat for each buffer distance)

- Open Geoprocessing and search for Buffer.

- Set the parameters as follows:

  - Input Features: Sites_No_ID

  - Output Feature Class:

    - For 20 m buffer: Sites_No_ID_Buffer_20m

    - For 50 m buffer: Sites_No_ID_Buffer_50m

  - Distance: 20 Meters or 50 Meters

  - Method: Planar

  - Dissolve Type: No Dissolve

- Run the tool.

Each buffer layer will contain one buffer polygon per site.

## 4. Visual Proximity Assessment

To evaluate spatial relationships between unidentified sites and sites with established identifiers, buffer layers were visually compared with the projected sites that contain valid IDs.

### Steps

- Add the following layers to the map:

  - Sites_No_ID_Buffer_20m

  - Sites_No_ID_Buffer_50m

  - Sites_With_Project_ID

- Inspect which sites from Sites_With_Project_ID fall within each buffer distance.

### Interpretation

- Sites within the 20 m buffer were interpreted as spatially coincident with unidentified sites and likely represent the same sampling location.

- Sites within the 50 m buffer were interpreted as spatially proximate but not necessarily identical locations.

## 5. Summary of Workflow

- Project all sampling site layers to StatePlane Florida North (meters).

- Isolate sites lacking organizational and monitoring identifiers.

- Assign a provisional site ID using the U of F Number.

- Create 20 m and 50 m buffers around unidentified sites.

- Visually inspect proximity between unidentified sites and identified sites to inform QA/QC and site reconciliation decisions.