# Overview Methods: Development of the Bays_Bayous_Lakes Boundary Layer

## Purpose

The Bays_Bayous_Lakes boundary layer was developed to create a standardized polygon dataset representing the major bays, bayous, lakes, and estuarine waters within the St. Andrew Bay Watershed.

## Source Datasets

The boundary layer was developed by integrating the following datasets:

National Hydrography Dataset Plus High Resolution (NHDPlus HR) waterbody dataset

Florida Department of Environmental Protection (FDEP) Florida Lakes dataset

Florida Numeric Nutrient Criteria (NNC) Estuary Nutrient Regions dataset

National Coastal Condition Assessment (NCCA) boundary layer

## Hydrographic Boundary Development

The source datasets were merged into a single working feature class. Because multiple datasets represented many of the same waterbodies, the merged layer contained overlapping polygons, duplicate boundaries, and inconsistencies that required refinement.

Boundary development consisted of multiple iterative rounds of geoprocessing and manual editing, including:

Merge to combine all source datasets into a single working layer.

Pairwise Clip to retain desired portions of overlapping polygons while removing redundant geometry.

Symmetrical Difference to identify conflicting geometry between datasets for review and refinement.

Dissolve to remove internal boundaries and combine polygons representing the same continuous waterbody.

Use the explode editing tool to separate multiple polygons with the same name and then rename the polygons correctly in the attribute table

Repeated processing as additional edits and refinements were incorporated.

Remove sliver polygons

Reassign incorrect bay segment assignments.

Address naming inconsistencies.

Additional clipping, dissolving, and manual editing as needed.